# LAB # 03

# Interrupts an Advanced I/O

# Spring 2026 CSE-420L Embedded Systems Lab

Submitted by: Naveed Ahmad

Registration No.: 22PWCSE2165

Class Section: B

“On my honor, as student at University of Engineering and Technology, I have neither given nor received unauthorized assistance on this academic work.”

Submitted to:

# Engr. Muhammad Shahzad

March 5th, 2026

Department of Computer Systems Engineering

University of Engineering and Technology, Peshawar

Objective:

- To understand the concept of polling and interrupts in microcontrollers.

- To study different types of interrupts.

- To analyze interrupt handling in the ATmega328P microcontroller.

- To learn the role of important interrupt registers and advanced I/O operations.

Theory:

Polling:

Polling is a technique where the microcontroller continuously checks the status of an input device to determine whether an event has occurred.

In polling, the processor repeatedly reads the status of the device in a loop. This method consumes more CPU time because the processor must constantly check the device even when no event occurs.

Interrupts are preferred over polling because they allow the processor to perform other tasks and respond only when an event happens.

Types of Interrupts:

Interrupts can be classified into the following types:

1. Hardware Interrupt:

A hardware interrupt is generated by an external hardware device connected to the microcontroller.

Example:

- Push button press

- Sensor signal

These interrupts are triggered through external interrupt pins such as INT0 and INT1 in the ATmega328P.

2. Software Interrupt:

A software interrupt is generated by a program instruction during program execution.

Example:

- System calls

- Special instructions that request processor attention.

- Internal Interrupt:

Internal interrupts are generated by internal peripherals of the microcontroller.

Examples:

  - Timer overflow

  - ADC conversion complete • 	USART transmission complete

- External Interrupt:

External interrupts are generated by signals coming from outside the microcontroller.

Example:

  - A push button connected to INT0 pin triggering an interrupt.

Internal Architecture of ATmega328P:

The ATmega328P is an 8-bit microcontroller based on the AVR architecture.

Main components of its architecture include:

  - CPU Core

  - Flash Memory (program storage) • 	SRAM (temporary data storage)

  - EEPROM (non-volatile memory)

  - I/O Ports

  - Timers/Counters

  - Interrupt Unit

The interrupt controller monitors interrupt signals and transfers control to the Interrupt Service Routine (ISR) when an interrupt occurs.

Important Registers:

- SREG (Status Register)

The Status Register (SREG) stores the status of arithmetic operations and processor state.

Important bit:

  - Global Interrupt Enable (I-bit) which enables or disables interrupts in the system.

- EIMSK (External Interrupt Mask Register):

The External Interrupt Mask Register enables or disables external interrupts.

Example:

  - Setting the INT0 bit enables interrupt from pin INT0.

- EICRA (External Interrupt Control Register A):

The EICRA register controls how external interrupts are triggered.

It defines the interrupt trigger type such as:

  - Low level

  - Logical change

  - Falling edge

  - Rising edge

- TIMSK (Timer Interrupt Mask Register):

The Timer Interrupt Mask Register enables or disables timer-related interrupts such as:

  - Timer overflow interrupt

  - Output compare interrupt Interrupt Execution Steps:

When an interrupt occurs in the ATmega328P, the following steps take place:

  - The microcontroller finishes executing the current instruction.

  - The program counter (PC) address is saved onto the stack.

  - The Global Interrupt Flag in the SREG register is cleared.

  - The processor jumps to the Interrupt Service Routine (ISR) address.

  - The ISR executes the required task.

  - After completion, the RET I instruction returns the program to the main execution.

Components / Registers Used:

The following registers and components are used in this experiment:

| Register | Function |
| --- | --- |
| DDRB | Configures PORTB pins as input or output |
| PORTB | Used to send output signals (e.g., LED control) |
| EIMSK | Enables or disables external interrupts |
| EICRA | Selects the interrupt triggering condition |
| SREG | Contains the global interrupt enable flag |

Code:

#include <avr/io.h> #include<avr/interrupt.h> volatile uint8_t count = 0;

ISR(TIMER0_OVF_vect)

{

count++;

if(count >= 31)

{

PORTB ^= (1 << PB0);       count=0;

}

}

int main()

{

DDRB |= (1 << PB0);

TCCR0B |= (1 << CS02)|(1 << CS00); //Clock select: 1024 Prescaler     TIMSK0 |= (1 << TOIE0); //enable timer overflow interrupt     sei();     while (1)

{

}

}

Circuit Diagram: