# Lab Report No 04

Performance Analysis of Embedded Programs

Spring 2026

CSE-420L Embedded Systems LAB

Submitted By: Naveed Ahmad

RegistrationNo:22PWCSE2154

Section: B

#### “On my honor, as student of University of Engineering and Technology, I have neither given nor received unauthorized assistance on this academic work”

Submitted to:

#### Engr. Muhammad Shahzad

March 17, 2026

## Department of Computer Systems Engineering University of Engineering and Technology Peshawar

Title:	Performance Analysis of Embedded Programs

Latency:

Latency is the time delay between giving a command and the system starting to respond.

Example:

In Arduino, the time between sending a signal to a pin and the pin actually changing state is called latency.

Latency = Response delay time

Throughput:

Throughput is the amount of work or number of operations completed in a given time. Example:

How many times Arduino can toggle a pin in one second.

Throughput = Number of operations per unit time

# Objective:

- To measure the execution time of pin toggling in Arduino Uno.

- To compare the speed of digitalWrite() function and Direct Register Access (PORTB).

- To understand how low-level register manipulation improves performance in embedded systems.

## Apparatus:

- Arduino Uno R3

- USB Cable

- Computer / Laptop

- Arduino IDE

- LED (optional)

# Theory:

Arduino boards use a microcontroller to control input and output pins. The execution time of a command shows how long the microcontroller takes to perform an operation.

- Using digitalWrite()

digitalWrite() is a high-level Arduino function used to set a pin HIGH or

LOW.

Example:

digitalWrite(13, HIGH); digitalWrite(13, LOW);

Advantages:

  - Easy to use

  - Portable across boards Disadvantages:

  - Slower because it performs multiple internal operations such as:

    - Checking pin mapping

    - Setting registers internally

### Using Direct Register Access

In this method we directly manipulate hardware registers of the microcontroller. Example registers:

  - DDRB → Data Direction Register

  - PORTB → Output Register

Example:

PORTB |= (1 << PORTB5); PORTB &= ~(1 << PORTB5);

Advantages:

  - Much faster execution

  - Less overhead

Disadvantages:

  - Harder to understand

  - Hardware dependent

### Execution Time Measurement

The function micros() is used to measure time in microseconds. Example:

startTime = micros();

After executing code:

executionTime = endTime - startTime;

## Circuit diagram Experiment 1:

### Connections:

  - Arduino Pin 13 → Resistor → LED → GND

Note: Arduino Uno also has a built-in LED on pin 13, so external LED is optional.

## Experiment 1: Using digitalWrite() Algorithm:

- Start the program.

- Initialize serial communication at 9600 baud.

- Set pin 13 as OUTPUT.

- Record start time using micros().

- Run a loop 1000 times.

- Set pin HIGH using digitalWrite().

- Set pin LOW using digitalWrite().

- Record end time using micros().

- Calculate execution time.

- Print execution time on Serial Monitor.

- Repeat after 2 seconds.

Source code: unsigned long startTime; unsigned long endTime;

unsigned long executionTime;

void setup() { Serial.begin(9600); pinMode(13, OUTPUT);

}

void loop() { startTime = micros();

for (int i = 0; i < 1000; i++) { digitalWrite(13, HIGH); digitalWrite(13, LOW);

}

endTime = micros();

executionTime = endTime - startTime;

Serial.print("Execution Time (digitalWrite): "); Serial.print(executionTime);

Serial.println(" us");

delay(2000);

}

## Experiment 2: Using Direct Register Access Algorithm:

- Start the program.

- Initialize Serial communication.

- Configure Pin 13 (PB5) as OUTPUT using DDRB register.

- Record start time.

- Run loop 1000 times.

- Set pin HIGH using PORTB.

- Set pin LOW using PORTB.

- Record end time.

- Calculate execution time.

- Display the result on Serial Monitor.

- Repeat after delay

Source code:

// Experiment 2: Execution time using Direct Register Access

unsigned long startTime; unsigned long endTime; unsigned long executionTime;

void setup() { Serial.begin(9600);

DDRB |= (1 << DDB5); // Set pin 13 (PB5) as OUTPUT

}

void loop() { startTime = micros();

for (int i = 0; i < 1000; i++) {

PORTB |= (1 << PORTB5); // Set pin HIGH PORTB &= ~(1 << PORTB5); // Set pin LOW

}

endTime = micros();

executionTime = endTime - startTime;

Serial.print("Execution Time (Registers): "); Serial.print(executionTime);

Serial.println(" us");

delay(2000);

}

## Procedure:

- Connect Arduino Uno to the computer using a USB cable.

- Open Arduino IDE.

- Write the first program using digitalWrite().

- Upload the code to Arduino.

- Open Serial Monitor.

- Observe the execution time results.

- Now write the second program using Direct Register Access.

- Upload the code again.

- Observe the execution time results.

- Compare both results.