LAB 05

# “Hardware Timers”

Spring 2026

Embedded Systems Lab

Submitted by: Naveed Ahmad

Registration No.: 22PWCSE2165

Class Section:B

Student Signature: ______________

Submitted to:

Engr. Muhammad Shahzad

April 22, 2026

Department of Computer Systems Engineering

University of Engineering and Technology, Peshawar

Lab : 05

# “Hardware Timers”

Timers:

- Crystal Oscillator / Pulse Generator  Quartz (16MHz)

ATmega328p

- Timers increment automatically with every clock tick

- Like 255 for 8-bit   ,   65535 for 16 bit

- Generate delays

- Measuring time Intervals

- Producing PWM Signals

- Triggering interrupts at precise intervals Time  =  timer count * prescaler  /  clock frequency

e-g:

Timer 0  =  8 bits  ->  millis , micros , delays

Timer 1  =  16 bits  ->  Servo libraries , High Resolution PWM

Timer 2  =   8 bits  ->   Tone Generation

Prescaler:

A numerical frequency divider placed before the timer , It extends the range of the timer so u can measure longer intervals or generate slower frequencies without using huge registers.

Ex:

System Clock = 16 MHz

Prescaler 1 = Timer tick after every 62.5 ns

Prescaler 8 = Timer tick after every 0.5 ns

Prescaler 64 = Timer tick after every 4 ns

Prescaler 1024 = Timer tick after every 64 ns

Without the prescaler the timer will overflow in 4ms with prescaler 1024

Max interval = 65536 * 64 micro S

= 4.2 Sec

TASK 1 :

LED blinks using hardware timers

Void setup(){

Pinmode(13,OUTPUT);

TCCR1A = 0;

TCCR1B = 0;

TCCR1B|= (1<<CS12) (1<<CS10);

}

Void loop(){

If (TCNT1>=15625){ TCNT1=0;

digitalwrite(13,!digitalRead(13));

}

}

 Setting PreScaler:

CS12 = 1

CS11 = 0

CS10 = 1

001 divide by 1

010 divide by 8

011 divide by 64

- divide by 256

- divide by 1024

Task picture :

Task 2 :

CODE :

void setup() {   pinMode(13, OUTPUT);

TCCR1A = 0;

TCCR1B = 0;

TCCR1B |= (1 << WGM12);                  // Enable CTC mode

OCR1A = 15624;                           // Compare value

TCCR1B |= (1 << CS12) | (1 << CS10);     // Prescaler = 1024

}

void loop() {   if (TIFR1 & (1 << OCF1A)) {              // Check compare match flag     TIFR1 |= (1 << OCF1A);                 // Clear flag     digitalWrite(13, !digitalRead(13));    // Toggle LED

}

}

Task picture:

Task 3 :

Code:

const int LED_PIN = 9;

volatile int brightness = 0; volatile int step = 10;

volatile int tickCount = 0; const int TICKS_PER_STEP = 6;

ISR(TIMER2_COMPA_vect) {

tickCount++;   if (tickCount < TICKS_PER_STEP) return;   tickCount = 0;   brightness += step;

if (brightness >= 999) {     brightness = 999;     step = -10;

} else if (brightness <= 0) {     brightness = 0;     step = 10;   }

OCR1A = brightness;

}

void setup() {

pinMode(LED_PIN, OUTPUT);

TCCR1A = (1 << COM1A1) | (1 << WGM11);

TCCR1B = (1 << WGM13) | (1 << WGM12) | (1 << CS11);

ICR1   = 999;   OCR1A  = 0;

TCCR2A = (1 << WGM21);

TCCR2B = (1 << CS22) | (1 << CS21) | (1 << CS20);

OCR2A  = 255;

TIMSK2 = (1 << OCIE2A);

sei();

}

void loop() {

}

void loop() {

}

Conclusion:

Hardware timers on the ATmega328p enable precise, CPU-independent time control through configurable prescalers and timer registers, allowing applications ranging from simple LED blinking with TCNT1 polling, to accurate CTC-mode compare interrupts, to smooth PWM-driven LED breathing — all without relying on software delays.