LAB 06

Analogue to Digital Conversion (ADC)

Spring 2026

CSE-420L Embedded Systems LAB

Submitted By:  Naveed Ahmad

Registration No:  22PWCSE2165

Section: B

“On my honor, as student of University of Engineering and Technology, I have neither given nor received unauthorized assistance on this academic work”

Submitted to:

Engr. Muhammad Shahzad

May 3, 2026

Department of Computer Systems Engineering

University of Engineering and Technology Peshawar

ADC:

ADC is the process of converting a continuous physical quantity (analog signal) into a digital representation (binary numbers). This process is governed by the Nyquist-Shannon Sampling Theorem, which states that to perfectly reconstruct an analog signal, you must sample it at a rate f_s at least twice as high as the highest frequency component present in the signal fmax

f_s > 2 fmax

If this condition is not met, aliasing occurs, where high-frequency components appear as lowfrequency artifacts in the sampled data.

Stages of Conversion

The conversion process typically involves three distinct stages:

- Sampling: Converting a continuous-time signal into a discrete-time signal by taking periodic "snapshots" of the amplitude at specific time intervals T_s = 1/f_s.

- Quantization: Mapping the infinite range of possible analog amplitudes to a finite set of discrete levels. Each level is determined by the ADC's resolution.

- Encoding: Assigning a unique binary code to each quantized level.

Common ADC Architectures

Depending on your lab equipment, you are likely using one of these common topologies:

- Flash ADC: Extremely fast but hardware-intensive (uses 2^n - 1 comparators). Best for high-speed applications.

- Successive Approximation Register (SAR) ADC: A balanced approach that uses a binary search algorithm to arrive at the digital value. It is the most common architecture for general-purpose microcontrollers.

- Delta-Sigma ADC: Uses oversampling and noise shaping. It provides very high resolution and is commonly used in high-fidelity audio applications.

TASK:1

Read Anaolgue value using potentiometer  and display ADC value on serial monitor.

CODE:

#include <avr/io.h>

#include <stdio.h>

#include <util/delay.h>

// Initialize USART for Serial communication at 9600 baud void USART_init(void) {

UBRR0H = 0;

UBRR0L = 103; // For 16MHz clock and 9600 baud

UCSR0B = (1 << TXEN0); // Enable Transmitter

UCSR0C = (1 << UCSZ01) | (1 << UCSZ00); // 8-bit data

}

// Send a character over USART void USART_transmit(char data) {     while (!(UCSR0A & (1 << UDRE0)));

UDR0 = data;

}

// Initialize ADC void ADC_init(void) {

// AREF = AVcc

ADMUX = (1 << REFS0);

// Enable ADC, set Prescaler to 128 (16MHz/128 = 125kHz)

ADCSRA = (1 << ADEN) | (1 << ADPS2) | (1 << ADPS1) | (1 << ADPS0);

}

// Read ADC value uint16_t ADC_read(uint8_t ch) {

ADMUX &= 0xF0; // Clear previous channel

ADMUX |= ch;   // Select new channel     ADCSRA |= (1 << ADSC); // Start conversion     while (ADCSRA & (1 << ADSC)); // Wait for conversion to complete     return ADC; // Return 10-bit result

}

int main(void) {

USART_init();

ADC_init();

char buffer[10];

while (1) {         uint16_t adc_val = ADC_read(0);

// Convert integer to string and transmit         sprintf(buffer, "%d\r\n", adc_val);         for(int i = 0; buffer[i] != '\0'; i++) {

USART_transmit(buffer[i]);

}

_delay_ms(500);

}

}

OUTPUT:

TASK:2

Control LED brightness based on sensor i/p(use hardware registers)

CODE:

#include <avr/io.h>

// Initialize ADC for 10-bit resolution void ADC_init(void) {

// REFS0: Set Reference Voltage to AVcc (5V)

// The MUX bits are left at 0 to select ADC Channel 0 (PC0)

ADMUX = (1 << REFS0);

// ADEN: Enable ADC

// ADPS2, ADPS1, ADPS0: Set Prescaler to 128 (16MHz/128 = 125kHz ADC clock)

ADCSRA = (1 << ADEN) | (1 << ADPS2) | (1 << ADPS1) | (1 << ADPS0); }

// Read from the currently selected ADC channel uint16_t ADC_read(void) {

ADCSRA |= (1 << ADSC); // Set ADSC bit to start the conversion     while (ADCSRA & (1 << ADSC)); // Wait until ADSC clears (conversion complete)     return ADC; // Return the 10-bit ADC result

}

// Initialize Timer1 for 10-bit Fast PWM void PWM_init(void) {

// Set PB1 (Pin 9 on Arduino Uno) as an output pin

DDRB |= (1 << PB1);

// TCCR1A Settings:

// COM1A1 = 1, COM1A0 = 0: Clear OC1A on Compare Match, set OC1A at BOTTOM (Noninverting mode)

// WGM11 = 1, WGM10 = 1: Fast PWM, 10-bit mode

TCCR1A = (1 << COM1A1) | (1 << WGM11) | (1 << WGM10);

// TCCR1B Settings:

// WGM12 = 1: Completes the 10-bit Fast PWM mode selection

// CS11 = 1, CS10 = 1: Set clock prescaler to 64 (Yields a PWM frequency of ~244 Hz at 16MHz)     TCCR1B = (1 << WGM12) | (1 << CS11) | (1 << CS10);

// Initialize Output Compare Register (Duty Cycle) to 0

OCR1A = 0;

}

int main(void) {     ADC_init();

PWM_init();

while (1) {

// 1. Read the 10-bit value from the sensor (0 to 1023)         uint16_t sensor_val = ADC_read();

// 2. Write the 10-bit value directly to Timer1's Compare Match register

// A value of 0 means 0% duty cycle (LED off)

// A value of 1023 means 100% duty cycle (LED fully on)

OCR1A = sensor_val;

}

}

OUTPUT:

Conclusion:

This lab successfully demonstrated the integration of Analoge-to-Digital Conversion (ADC) and Pulse

Width Modulation (PWM) using AVR hardware registers. By interfacing a potentiometer with the ATmega328P, we effectively transformed continuous analog signals into 10-bit digital values, verifying the process through serial monitoring. Furthermore, applying these digital values to control LED brightness showcased a practical application of data acquisition and hardware control, reinforcing the importance of sampling rates, resolution, and register-level programming in embedded systems design.