# LAB 09

I2C Communication

### Spring 2026

Embedded Systems Lab

Submitted By: Naveed Ahmad

Registration No: 22PWCSE2165

Class Section: B

Submitted to:

### Engr. Muhammad Shahzad

June 25, 2026

Department of Computer Systems Engineering

University of Engineering and Technology

LAB 09

I2C COMMUNICATION (inter-integrated  service)

I2C is a simple, low-speed, two-wire serial communication protocol used to connect multiple microcontrollers, sensors, displays, memory chips, and other peripherals on the same circuit board. Think of it as a bus where all devices share the same two wires.

### The Two Essential Wires

| Wire Name | Purpose |
| --- | --- |
| SDA (Serial Data Line) | Carries the actual data being sent (bi-directional). |
| SCL (Serial Clock Line) | Carries the clock signal generated by the master to synchronize data transfer. |

Pull-up resistors are required on both wires (typically 4.7kΩ) to pull the lines to a logic high when no device is driving them.

How I2C Works (Master-Slave)

I2C uses a Master/Slave architecture.

Master: The device that controls the clock (SCL) and initiates all communication. (e.g., a microcontroller).

Slave: A peripheral device that waits for commands from the master. (e.g., temperature sensor)

Key feature: Every slave device has a unique 7-bit or 10-bit address. The master uses this address to talk to a specific slave.

Working: Every device has a unique address(like our reg.no), Send a start signal, broadcast the address, says read\write, the matching device responds, sends a stop signals

### Pros vs. Cons

| Pros | Cons |
| --- | --- |
| Only 2 wires (SDA, SCL) for many devices | Slower than SPI (typically 100kbps, 400kbps, or 3.4Mbps) |
| Multi-master & Multi-slave support | Requires pull-up resistors, limiting cable length |
| Simple addressing – no chip-select pins | Devices share the bus, so collisions are possible |
| ACK/NACK handshake for error checking | Power consumption higher than SPI due to pull-ups |

Uses

Reading temperature/humidity sensors (e.g., BME280, AHT20)

Communicating with OLED/LCD displays (SSD1306)

Interfacing with real-time clocks (DS3231)

Reading EEPROM memory chips (24LCxx)

Connecting multiple Arduinos or micro:bits

HALF-DUPLEX COMMUNICATION

TASK 01:

- Scan I2C bus (find connected device). write a program that checks all 7-bit I2C adresses (0-127)

- For each address .>send start, >send address ,>check for ACK

TASK 2

- Master writes data to slave 2x Arduino

MASTER WRITES

SLAVE RECEIVES

TASK 03:

Master reads data from slave

# Master Reads :

:

Slave Write: