Lab 13 EDGE AI

Spring 2026

CSE 420L: Embedded Systems Lab

Submitted by: Naveed Ahmad

Registration No.: 22PWCSe2165

Class Section: B

Submitted to:

Engr. Muhammad Shahzad

25 June 2026

Department of Computer Systems Engineering

University of Engineering and Technology, Peshawar

OBJECTIVES:

 To deploy ML models (TFLite) on ESP32 for on-device inference.

 To build and test a regression model and an anomaly detection model on edge hardware.

THEORY:

- Edge Devices

Edge devices are computing systems that execute Artificial Intelligence (AI) models directly on the hardware where data is generated, instead of sending the data to a remote cloud server. This approach significantly reduces response time (latency), minimizes bandwidth usage, improves privacy, and enables AI applications to function even without an internet connection.

- Common Edge Devices:

| Device | Memory / Features |
| --- | --- |
| Arduino Uno | ~2 KB RAM |
| STM32 | 64–512 KB RAM |
| ESP32 | 520 KB RAM |
| Raspberry Pi | 1 GB+ RAM |
| Smartphones | Several GB RAM |
| Smart Cameras | Built-in AI accelerators |
| NVIDIA Jetson Nano | GPU-based, suitable for deep learning |

### Applications of Edge Devices

Edge AI is widely used in many real-world applications, including:

- Smart agriculture (crop monitoring, automated irrigation)

- Smart home automation

- Industrial automation

- Healthcare monitoring

- Smart surveillance systems

- Environmental monitoring

## Edge AI Deployment Pipeline

Deploying a machine learning model onto a microcontroller follows these steps:

- Collect Data: Gather sensor or environmental data for training.

- Train the Model: Train a neural network using frameworks such as TensorFlow or Keras.

- Convert to TensorFlow Lite: Convert the trained model into TensorFlow Lite (TFLite) format. For microcontrollers, use TensorFlow Lite Micro.

- Quantization (INT8): Convert 32-bit floating-point weights into 8-bit integers. This reduces memory usage and improves inference speed.

- Convert to C Header File: Convert the .tflite model into a C array stored as a .h header file.

- Flash the Firmware: Upload the firmware containing both the AI model and application code to the ESP32.

- Run Inference: The microcontroller performs predictions using live sensor data.

## TensorFlow Lite Micro (TFLite Micro)

TensorFlow Lite Micro is a lightweight runtime specifically designed for microcontrollers. It operates entirely using static memory, making it suitable for devices with limited RAM and without an operating system.

### Supported Neural Network Layers

- Dense (Fully Connected)

- Conv2D

- LSTM

- Depthwise Convolution

### Arduino Library

- Arduino_TensorFlowLite

Alternatives:

- Eloquent TinyML

- Edge impulse(browser based)

## USB-to-Serial Bridge Chips

Most ESP32 development boards communicate with a computer using USB-to-UART bridge chips.

Common chips include:

- CP2102 (Silicon Labs)

- CH340 (WCH)

The correct driver must be installed for the Arduino IDE to recognize the ESP32 board.

- Important settings

- After dev modelue ep32

- Upload speed..921600

- Change frequency..240Mhz

- Flash size…4mb

- Partition scheme…Hugeapp

- Code dubug..none

- Install Libraries:

- Eloquent TinyMl

- AdafruitMPU6050

- AdafruitBME280

- Adafruit Unified sensor

TASK: 01

CODE:

- Step 1 — Train the Model (Google Colab / Python)

- Step 2 — Convert to Quantized TFLite Model

- Step 3 — Convert .tflite to a C Header File

- Step 4 — Deploy and Run Inference (Arduino / ESP32)

OUTPUT OF TASK 01:

TASK: 02

CODE:

- Step 1 — Record Normal Sensor Data (Arduino, run once)

- Step 2 — Train the Autoencoder (Google Colab / Python)

- Step 3 — Convert to TFLite Model

- Step 4 — Convert .tflite to a C Header File

- Step 5 — Deploy: Detect Anomalies and Trigger LED (Arduino / ESP32)

OUTPUT OF TASK 02:

CONCLUSION:

This lab demonstrated the complete edge-AI deployment pipeline, from training a model in Colab to running quantized inference on an ESP32. Task 1 showed accurate sine wave prediction using a regression model, while Task 2 used an autoencoder to successfully detect anomalies via reconstruction error. Together, they confirmed that even resource-constrained microcontrollers can run real-time ML inference effectively.