Assignment No 02

Course: CSE-420 Embedded Systems

Submitted By: Naveed Ahmad

Registration No: 22PWCSE2165

Section: B

“On my honor, as student of University of Engineering and Technology, I have neither given nor received unauthorized assistance on this academic work”

Signature: __________

Date: May 21, 2026

Submitted To: Engr. M. Asghar

Department of Computer Systems Engineering

University of Engineering and Technology

Bottom of Form

# UART CONNECTION STATEMENTS

- PD1 of the Master should be connected to PD1 of the Slave.
This statement is incorrect because PD1 is the TXD pin. In UART communication, the transmitter of one microcontroller must connect to the receiver of the other microcontroller.

- PD0 of the Master should be connected to PD0 of the Slave.
This statement is also incorrect because PD0 is the RXD pin. RXD pins cannot communicate directly with each other.

- A common ground connection is required between both microcontrollers.
This statement is correct because both ATMEGA328 microcontrollers must share the same voltage reference for proper UART communication.

- UART communication will not work if cross connections are not used.
This statement is correct because TXD must connect to RXD and RXD must connect to TXD. Without this cross connection, data transmission cannot occur properly.

Correct UART connections are:

- TXD (PD1) of Master → RXD (PD0) of Slave

- RXD (PD0) of Master → TXD (PD1) of Slave

# (B) CALCULATION OF UBRR FOR 9600 BPS

Given:

- CPU frequency = 16 MHz

- Baud rate = 9600 bps

- Asynchronous normal mode

- U2X = 0

The formula for normal mode is:

UBRR=FCPU16×Baud-1

Substituting the values:

UBRR=16,000,00016×9600-1
UBRR=16,000,000153,600-1
UBRR=104.1667-1
UBRR=103.1667

Since UBRR must be an integer value:

UBRR=103

# (c) Actual Baud Rate and Percentage Error

The formula for actual baud rate is:

Baudactual=FCPU16(UBRR+1)

Substituting the values:

Baudactual=16,000,00016(103+1)
Baudactual=16,000,0001664
Baudactual≈9615.38 bps

Now calculating percentage error:

%Error=9615.38-96009600×100
%Error≈0.16%

Therefore:

- Actual baud rate ≈ 9615.38 bps

- Percentage error ≈ 0.16%

# (D) DOUBLE SPEED MODE (U2X = 1)

For double speed mode, the formula becomes:

UBRR=FCPU8×Baud-1

Substituting the values:

UBRR=16,000,0008×9600-1
UBRR=16,000,00076,800-1
UBRR=208.3333-1
UBRR=207.3333

Taking the integer value:

UBRR=207

The actual baud rate is:

Baudactual=FCPU8(UBRR+1)

Baudactual=16,000,0008(207+1)
Baudactual=16,000,0001664
Baudactual≈9615.38 bps

Percentage error:

%Error≈0.16%

Hence:

- UBRR = 207

- Actual baud rate ≈ 9615.38 bps

- Percentage error ≈ 0.16%

# (E) UART FRAME FOR 0X5A

The hexadecimal value:

0x5A=010110102

UART sends data starting from the Least Significant Bit (LSB).

Therefore, the transmitted bit order becomes:

0 1 0 1 1 0 1 0

The UART frame contains:

- 1 Start bit

- 8 Data bits

- No parity bit

- 1 Stop bit

The complete transmitted frame is:

| Start Bit | Data Bits | Stop Bit |
| --- | --- | --- |
| 0 | 0 1 0 1 1 0 1 0 | 1 |

Final serial transmission sequence:

0 0 1 0 1 1 0 1 0 1